# Washstand Mesa70-M made from the copolymer-bound mineral material VARICOR® (material in accordance to ISO norm 19712). Homogeneous, coloured throughout, pore-free material, without surface sealing like gel coat or glaze, largely resistant against disinfectants and commonly-used chemicals, heat resistant against burning cigarette, standard plain white colour shades (arktis, white-alpine) have good antibacterial properties in accordance to ISO 22196 (other colours on request). Optionally available with antiviral properties.

Single washstand moulded to order in one piece, multiple washstand made of several single washstands bonded together seamlessly and ready made-up, with side shelves, machinable on site, with rectangular basin, standard without overflow, optionally possible with external overflow, optional tap hole, with skirt, with chamfered wall-connection profile, to be layed on a furniture or wall mounted.

Mesa70-M meets the requirements of the CE standard in compliance with DIN EN 14688.

Internal basin dimensions (L x W x D): 	700 x 310 x 100 mm

Depth:	500 mm

Material thickness:	12 mm

Skirt:	H = 50 mm

Chamfered wall-connection profile: 	H = 5 mm

Length of single washstand: 	800 – 2500 mm

Length of multiple washstand: 	from 1600 mm

Attachment: 	To be layed on a furniture or wall mounted

For wall mounting: 407 mm angle bracket

480-mm wall-mounting bracket for alcove installation

(available as additional accessories)

Accessories (optional):		• Permanent drain with VARICOR® cover

(for basins without overflow)

• Push-open with VARICOR® cover

(for basins with overflow)

• 407 mm angle bracket

• 480-mm wall-mounting bracket for alcove installation

Optional features:		• Side section, seamlessly bonded left and/or right

• Paper waste tube (round or rectangular), seamlessly

bonded

Model:	Mesa70-M

Distribution via:	VARICOR GmbH

Waldstr. 33

76571 Gaggenau (Germany)

Tel.: +49 (0) 7225/9739-0

www.varicor.com